# REMOTE SENSING

Comprehensive Academic Notes

# Table of Contents

1. Introduction to Remote Sensing

2. Remote Sensing Process and Components

3. Comparison: Satellite Remote Sensing vs. Aerial Photography

4. Advantages of Remote Sensing

5. Data Quality in Remote Sensing

6. Snow Detection Types

7. Resolutions in Remote Sensing

8. High-Quality Data Sources

9. Surface Reflectance and NDSI

10. Satellite Bands and Their Applications

11. Sentinel-1 Satellite

12. SAR (Synthetic Aperture Radar)

13. Polarization Modes

14. Google Earth Engine (GEE)

# 1. Introduction to Remote Sensing

## 1.1 Definition

Remote sensing is a technique of acquiring information about the Earth's surface without actually having physical contact with it. It involves sensing and recording reflected or emitted energy and processing, analyzing, and applying that information.

## 1.2 Basic Principle

Remote sensing works by detecting and measuring electromagnetic energy that is reflected or emitted from objects on Earth. This data is collected by sensors mounted on various platforms such as satellites or aircraft.

# 2. Remote Sensing Process and Components

## 2.1 Components of Remote Sensing System

A typical remote sensing system consists of the following components:

A. Energy Source (Sun)

Primary energy source for most remote sensing applications

Provides electromagnetic radiation

B. Satellite

Platform carrying sensors

Orbits Earth to collect data

C. Sensor

Detects and records electromagnetic radiation

Two main types: passive and active sensors

D. Earth Surface (Target)

Object or area being observed

Reflects or emits electromagnetic energy

E. Atmosphere

Medium through which energy travels

Can affect the quality of data collected

## 2.2 Remote Sensing Process Steps

The remote sensing process follows these sequential steps:

Step 1: Field Acquisition
For remote sensing to have energy source which illuminates or provides electromagnetic energy to the target of interest.

Step 2: Radiation and Atmosphere
As energy travels from its source to the target, it will come in contact with and interact with the atmosphere it passes through. This is called atmospheric interaction.

Step 3: Interaction with Target
Once energy reaches the target through the atmosphere, it interacts with the target depending on properties of both target and radiation.

Step 4: Recording of Energy by the Sensor
Sensors collect and record the electromagnetic radiation from target.

Step 5: Transmission, Reception, and Processing
Energy recorded by sensors has to be transmitted, often in electronic form, to a receiving and processing station where the data are processed into an image.

Step 6: Interpretation and Analysis
Processed data is interpreted, visually or digitally, or electronically, to extract information about the target which was illuminated.

Step 7: Application
Final element is achieved by applying the information we have been able to extract from the imagery about the target in order to better understand it, reveal some new information, or assist in solving a particular problem.

# 3. Comparison: Satellite Remote Sensing vs. Aerial Photography

Both satellite remote sensing and aerial photography are used for Earth observation, but they have distinct characteristics:

| Parameter | Satellite Remote Sensing | Aerial Photography |
| --- | --- | --- |
| Speed | Very high; high temporal resolution | Slower; needs more time to cover areas |
| Level of Detail | Scanners (5-20m resolution) | May go up to 2.5 cm |
| Weather Conditions | Cloud is big handicap; moderate condition | May work in high cloud/hazy cloud condition |
| Cost | Less broader | Costly (broader images cannot be taken) |
| Post Processing | Only band sample image covering large area | Tougher due to large number of images of small area |
| Location | Easy broader | Costly because images cannot be taken |

# 4. Advantages of Remote Sensing

Remote sensing offers several significant advantages over traditional ground-based methods:

Synoptic View: Provides a comprehensive overview of large areas

Global Coverage: Can monitor remote and inaccessible regions

Repeatability: Regular and systematic data collection

Multi-Resolution: Data available at various spatial and temporal resolutions

Multi-Platform: Sensors can measure energy at wavelengths beyond range of human vision

Near Real-time: Data can be collected and processed quickly for time-sensitive applications

Systematic Data Collection: Ensures consistent and standardized data

# 5. Data Quality in Remote Sensing

## 5.1 Sensor Logic vs. Data Quality

There are two main approaches to understanding snow and cloud detection in remote sensing:

A. The Optical Physics - Snow vs. Clouds
In the visible spectrum, snow and clouds both have high albedo (reflectance). Ice crystals are highly absorptive in SWIR (Shortwave Infrared), so on a SWIR satellite image, snow will not "glow", while bright clouds will be bright.

B. The Radar Physics - Dry vs. Wet Snow
When it's storming, we use SAR (Synthetic Aperture Radar). Radar waves pass right through dry snow like it's not even there, while wet snow appears bright to radars.

## 5.2 NDSI (Normalized Difference Snow Index)

NDSI is a commonly used index for snow detection that uses the reflectance properties of snow:

Green: Uses C-band microwave signal

Ocean: SWIR (Short-Wave Infrared)

Ocean + SWIR: Combined approach for better accuracy

Formula and Thresholds:
If NDSI is high (close to 1.0), it's definitely snow. If it's low, it's "cloud or bare ground" which allows to mask clouds.

NDSI = Masked: Means there is snow to count (0.4 to 1.0) range

NDSI = 1: Purest possible spectral of snow

SWIR = 0, so all specific value = 1

# 6. Snow Detection Types

## 6.1 Introduction

Snow detection in remote sensing can be categorized into different types based on the detection method and snow characteristics.

## 6.2 Snow Metamorphism ("Tracking Snow")

Fresh Snow: Small star-shaped crystals, very high surface area & reflect NIR.

Old (Slab) Snow: Sharper point blunts & grains generalize.

NIR Drop: Larger grains absorb more NIR light. By monitoring it for 3-5 days, weak layer or heavy slab is forming.

## 6.3 Ghost Types

Dry Snow: Radar Waves (C-band or L-band) pass right through dry snow like it's not even there.

Wet Snow: When snow melts, liquid water appears between the grains, dielectric absorbs radar energy.

## 6.4 Backscatter Drop

SAR imagery wet zone → dark as signals don't bounce back. Used to predict wet snow avalanches.

# 7. Resolutions in Remote Sensing

Remote sensing data is characterized by four types of resolutions:

## 7.1 Spatial Resolution (Pixel Size)

Smallest area on the ground.

Coarse Res (1km): Global weather phenomena area, mountain

Medium (10m-30m): Sentinel (all 10m) or Landsat (30m) → larger avalanche paths & snow covered slopes

High (30cm-4m): WorldView, Int. Vehicles or Small avalanche debris

## 7.2 Spectral Resolution (Color Detail)

Ability of a sensor to see specific wavelengths of light:

Multispectral: Sensors with 3 to 15 bands (colors). Used in NDSI

Hyperspectral: Sensors with hundreds of very narrow bands

## 7.3 Temporal Resolution (Repeat Rate)

Constellations used by Sentinel-2 & D2B together. Critical during active blizzards to track rapid accumulation of snow that tends to avalanches.

## 7.4 Radiometric Resolution (Brightness Sensitivity)

Sensor's sensitivity to small differences in energy:

8 bits (D levels): SAR imagery: Snow very bright & work out into a simple white blob

12-16 bits: Highly radiometric resolution. Sensitive to see the texture

# 8. High-Quality Data Sources

## 8.1 ISRO Bhuvan & Bhumidhri

Indian-Specific datasets like Cartosat-1 (DEMs), Resourcesat (LISS-III/IV) & OceanSat.

## 8.2 Copernicus Data Space Ecosystem (WorldView)

ESA (European Space Agency) datasets:

Sentinel-1 (SAR): Essential for Himalayan clouds

Sentinel-2 (Optical): 10m-20m resolution, perfect for NDSI calculation & monitor snow

Copernicus browser

## 8.3 USGS Earth Explorer & WorldView

Gold standard for historic & Global data:

Landsat 8 & 9: Long-term detection

DEMs (Digital Elevation Models)

Decrease bulk downloads openly

## 8.4 NASA Earthdata & WorldView - NRT mode

NRT (Near Real-Time) monitors for immediate data access.

# 9. Surface Reflectance and NDSI

## 9.1 TOA (Top of Atmosphere) Reflectance

TOA saves data; sensor records all the light that hits it after travelling entire atmosphere - light reflected from ground + reflected by clouds + gases.

## 9.2 SR (Surface Reflectance)

Corrected data. Complex masks to subtract interference of atmosphere.

## 9.3 TOA vs SR Comparison

| Aspect | TOA (Top of Atmosphere) | SR (Surface Reflectance) |
| --- | --- | --- |
| NDSI calculation | Masked (0.4 to 1.0) | NDSI = 1 |
| Interpretation | Means there is snow to count (0.4 to 1.0) range | Purest possible spectral of snow, SWIR = 0 so all specific value = 1 |

# 10. Satellite Bands and Their Applications

## 10.1 True Colours (Visible Spectrum)

| Band | Colour | Wavelength |
| --- | --- | --- |
| B2 | Blue | 490 nm |
| B3 | Green | 560 nm |
| B4 | Red | 665 nm |

## 10.2 Infrared Bands

| Band | Type | Description | Application |
| --- | --- | --- | --- |
| B8 | NIR | Near-Infrared | Snow detection |
| B11 | SWIR | Shortwave Infrared | Fire & snow |
| B12 | SWIR | Shortwave Infrared | Fire detection |
| B8A | Narrow NIR | Narrow Near-Infrared | Vegetation |

# 11. Sentinel-1 Satellite

## 11.1 Overview

Sentinel-1 is an active microwave emission SAR (Synthetic Aperture Radar) satellite that uses C-band microwave signal.

## 11.2 Key Features

Penetrates clouds & darkness (λ = 5.6 cm)

Backscatter & Interaction: Rough surfaces return energy back to satellite

Smooth surfaces reflect away from satellite (appear dark)

Synthetic Aperture Radar: Motion-based imaging technique

As satellite flies along path, it rapidly fires radar and records the echoes

Captures the same spot on ground from multiple positions

Math combining multiple echoes simulates an enormously large antenna (2m antenna equivalent)

## 11.3 Repeat Cycle

Repeat cycle: 12 days

# 12. SAR (Synthetic Aperture Radar) - Detailed

## 12.1 Modes

IW (Interferometric Wide Mode): Steps mode

EW (Extra Wide): Swory mode

Stripmap (SM): Specific mode

Wave (WV): SRS mode

## 12.2 Polarization

Orientation of electromagnetic wave:

VV → Vertical transmission; Vertical reception

VH → Vertical transmission; Horizontal reception (Cross-signal physical signal)

Smooth surfaces maintain polarization (store VV)

Rough vegetation scatters signal (cross VH)

## 12.3 GRD (Ground Range Detected)

Descending: IN | from N to S

Ascending: TOL | from S to N

# 13. Polarization Modes in Detail

## 13.1 RCS (Radar Cross Section)

No cloud filtering in Sentinel-1. dB = 10log₁₀ (Power) → dB is used to compress the range for easy visualization.

0dB → Strong reflector (rough, urban)
+10dB → Moderate
-20dB → Weak
< -30dB → Very weak

## 13.2 Interpretation

Bright areas → High backscatter
Dark → Low backscatter
Gray → Moderate

## 13.3 Surface Analysis

Smooth Surfaces → High Ratio VV Strong
Rough Surfaces → Low ratio VH strong

Ratio 7+ → Very smooth (Dry Snow → High Ratio)
3-7 → Moderately smooth (Wet Snow → Low Ratio)
1-3 → Rough
< 1 → Unusual

# 14. Google Earth Engine (GEE)

## 14.1 Overview

Google Earth Engine is a cloud-based platform for planetary-scale geospatial analysis.

## 14.2 Core Importance

Data Acquisition

Pre-processing

Index calculation

Terrain Integration

Parameters & Marking

Export & Validation

## 14.3 Properties

No Data Divides

Parallel Computing

Multi-Source Fusion

## 14.4 Analysis Features

Analysis - only data required

Cloud-native processing

API-based access

Public data catalog

# Conclusion

Remote sensing is a powerful and versatile technology that has revolutionized our ability to monitor and understand the Earth's surface. From snow detection in mountainous regions to global climate monitoring, remote sensing provides critical data for decision-making across various domains including environmental management, disaster response, agriculture, and urban planning.

The integration of multiple sensor types (optical and radar), various resolutions (spatial, spectral, temporal, and radiometric), and advanced processing platforms like Google Earth Engine has made remote sensing an indispensable tool for modern scientific research and practical applications.